SAFETY EYE GUARD

Prescription Safety Eyewear Program

Policy Template

A customisable written policy for employers running a company-funded prescription safety eyewear program. Replace every highlighted placeholder with your own detail, delete anything that does not apply, and have the finished policy reviewed by whoever owns EHS and HR at your organisation.

How to use this template

Placeholders appear like [COMPANY NAME] in orange. Every one needs replacing.

Sections 3 and 6 carry the regulatory detail most policies get wrong - read those before editing.

Appendix A is a hazard assessment worksheet. Complete one row per work area or task.

This template is a drafting aid. It is not legal advice, and it does not replace your own hazard assessment or a review against the rules that apply to your sites.

1. Purpose

The purpose of this policy is to protect the eyesight of employees at [COMPANY NAME] who require corrective lenses and who work in areas where eye hazards are present. It sets out who is eligible for prescription safety eyewear, what specification that eyewear must meet, how it is obtained and replaced, and how compliance is monitored.

2. Scope

This policy applies to all employees, and to contractors and visitors where stated, at [SITE / LOCATION LIST]. It covers prescription safety eyewear only. Non-prescription protective eyewear, face shields, welding helmets and laser eyewear are covered by [RELATED PPE POLICY REFERENCE].

3. Regulatory basis

This policy is written to meet the following requirements. Confirm which apply to each of your sites.

OSHA 29 CFR 1910.133 - eye and face protection, general industry.

OSHA 29 CFR 1926.102 - eye and face protection, construction.

OSHA 29 CFR 1910.132(d) - the employer must assess the workplace to determine whether hazards requiring PPE are present.

ANSI/ISEA Z87.1 - the consensus standard defining performance and marking for eye and face protection.

Employees who wear prescription lenses and are exposed to eye hazards must either wear protective eyewear that incorporates their prescription, or wear protection that fits properly over their prescription eyewear without disturbing either item.

Prescription protective eyewear must be built on a frame marked Z87-2. Ordinary optical frames fitted with toughened lenses are not compliant, and clip-on side shields do not make them compliant.

4. Definitions and markings

Markings are stamped on the frame and lens and are the practical way to verify compliance at point of issue.

| Marking | Meaning | When it is required |
| --- | --- | --- |
| Z87 | Basic impact rating | Minimum for general eye hazard areas |
| Z87+ | High impact rating | Flying particles, grinding, chipping, fastening |
| Z87-2 | Prescription frame marking | All prescription protective eyewear, without exception |
| D3 | Splash and droplet | Chemicals, solvents, fluids, sanitation chemistry |
| D4 | Dust | Airborne particulate, drilling, dry material handling |
| D5 | Fine dust | Fine or continuous particulate exposure |
| W + shade no. | Welding filter shade | Welding, cutting and brazing, shade matched to process |
| U + scale no. | UV filter scale | Sustained outdoor work |

5. Eligibility

An employee is eligible under this policy where both of the following are true: they hold a current prescription for corrective lenses, and their role requires them to enter or work in an area identified in the hazard assessment as requiring eye protection. Eligibility is confirmed by [ROLE RESPONSIBLE - e.g. EHS Manager].

Contractors and visitors entering hazard areas [ARE / ARE NOT] covered by this policy and must [PROVIDE THEIR OWN COMPLIANT EYEWEAR / BE ISSUED EYEWEAR ON ARRIVAL].

6. Cost and payment

Under OSHA 29 CFR 1910.132(h) the employer must pay for required PPE. There is one specific exemption relevant to this policy: the employer is not required to pay for non-specialty prescription safety eyewear, provided the employer permits such eyewear to be worn off the job site.

Specialty eyewear - prescription inserts, sealed or gasketed frames, welding filters, and any eyewear that cannot reasonably be worn away from site - is not covered by that exemption and remains employer-paid.

[COMPANY NAME] will fund prescription safety eyewear as follows: an allowance of [AMOUNT] per eligible employee every [PERIOD], covering [WHAT IS INCLUDED - frame, lenses, coatings]. Costs above the allowance are [EMPLOYEE-FUNDED / APPROVED CASE BY CASE].

Note for drafting: a funded allowance is generally recommended even where the exemption applies, because cost is the most common reason employees continue wearing non-compliant personal eyewear.

7. Prescriptions and eye examinations

Employees must supply a current prescription, dated within [NUMBER] months. [COMPANY NAME] [DOES / DOES NOT] fund the eye examination itself. Prescriptions are re-confirmed every [PERIOD], or sooner where an employee reports a change in vision.

Employees must report any change in vision that affects their ability to work safely.

Pupillary distance is required in addition to the prescription for accurate lens fitting.

Employees requiring progressive or multifocal lenses should declare the tasks involved, as standard progressive designs perform poorly for overhead work.

8. Eyewear specification by hazard

The specification below is the minimum for each hazard type. Where more than one hazard is present, the requirements combine.

| Hazard present | Minimum marking | Specification notes |
| --- | --- | --- |
| Flying particles, grinding, chipping | Z87+ | Polycarbonate lens with permanent side protection |
| Chemical or fluid splash | Z87+ / D3 | Sealed frame, or goggle over prescription eyewear for high-risk transfers |
| Airborne dust | Z87+ / D4 | Foam-gasketed or wrap frame maintaining a seal |
| Fine or continuous particulate | Z87+ / D5 | Fully sealed frame; anti-fog essential due to restricted airflow |
| Welding, cutting, brazing | W + shade | Filter shade matched to process and amperage; impact eyewear worn underneath |
| Sustained outdoor work | Z87+ / U6 | Tinted or photochromic; tint darkness is not UV protection |
| Temperature or humidity transitions | Z87+ | Permanent bonded anti-fog, not a wipe-on treatment |

9. Ordering process

Eligible employees order through [ORDERING ROUTE - e.g. the company safety eyewear portal]. The process is:

Employee confirms eligibility and obtains a current prescription.

Employee selects a frame from the approved list for their hazard area.

Order is approved by [APPROVER ROLE] and placed with the supplier.

Eyewear is delivered to [DELIVERY POINT] within [EXPECTED LEAD TIME].

Fit is checked at issue, and the issue is recorded against the employee.

10. Replacement, damage and loss

Eyewear damaged in the course of work is replaced at company cost. Eyewear lost or damaged through negligence is [REPLACED AT COMPANY COST / EMPLOYEE-FUNDED]. Replacement is requested through the same route as a new order, and interim protection must be issued where an employee is left without compliant eyewear.

Scratched or pitted lenses that impair vision must be replaced rather than kept in service. Degraded lenses are a hazard in their own right.

11. Compliance, monitoring and records

[ROLE RESPONSIBLE] maintains a record of every employee issued prescription safety eyewear, the specification issued, and the date. Records are retained for [RETENTION PERIOD].

Eyewear use is checked during routine area inspections and documented.

Non-compliance is addressed under the company disciplinary procedure.

The hazard assessment underpinning this policy is reviewed every [PERIOD], and whenever a process, material or work area changes.

12. Roles and responsibilities

| Role | Responsibility |
| --- | --- |
| [EHS / SAFETY MANAGER] | Owns the hazard assessment, sets the specification, monitors compliance |
| [LINE MANAGER / SUPERVISOR] | Confirms eligibility, checks eyewear is worn and serviceable |
| [HR / BENEFITS] | Administers allowances and eligibility records |
| Employee | Wears the eyewear issued, reports damage and vision changes, keeps prescription current |
| [SUPPLIER] | Supplies compliant marked eyewear and maintains order records |

Appendix A - Hazard assessment worksheet

Complete one row per work area or task. This worksheet supports the assessment required by OSHA 29 CFR 1910.132(d).

| Area / task | Hazards present | Required marking | Eyewear specified | Assessed by | Date |
| --- | --- | --- | --- | --- | --- |

Prepared by Safety Eye Guard - prescription and non-prescription safety eyewear programs. safetyeyeguard.com

This template is a drafting aid, not legal advice. Review the finished policy against the regulations applying to your sites before adopting it.